Instruction Manual

G259

5.1 channel (1080p) HDMI DVD Player

# Contents

1.	Precautions----------------------------------------------------------------------------------------------4

2.	Front Panel and Rear Panel Illustration ---------------------------------------------------------5

3.	Remote Control-----------------------------------------------------------------------------------------6

4.	Connections---------------------------------------------------------------------------------------------7

5.	Function Operation------------------------------------------------------------------------------------8

6.	Playing WMA, MP3 and JPEG discs -----------------------------------------------------------10

7.	CD-RIPPING -----------------------------------------------------------------------------------------11

8.	Setup Menu System --------------------------------------------------------------------------------12

9.	Specification ------------------------------------------------------------------------------------------16

10 .  Troubleshooting----------------------------------------------------------------------------------------17

WARNING:  TO REDUCE THE RISK OF FIRE OR ELECTRIC SHOCK, DO NOT EXPOSE THIS APPLIANCE TO RAIN OR MOISTURE.

CAUTION: TO REDUCE THE RISK OF ELECTRIC SHOCK, DO NOT REMOVE COVER (OR BACK). NO USER SERVICEABLE PARTS INSIDE. REFER SERVICING TO  QUALIFIED SERVICE PERSONNEL.

The lightening flash with arrowhead symbol, within an equilateral triangle, is intended the alert the user to the presence of uninsulated dangerous voltage within the products enclosure that may be of sufficient magnitude to constitute a risk of electric shock to persons.

The exclamation point within an equilateral triangle is intended to alert user to the presence of important operating and maintenance (servicing) instructions in the literature accompanying the appliance.

Laser product symbol: located on the rear panel of the unit, indicate this player is a class

laser product, only use weak laser and no hazardous radiation outside of the unit.

1.Read through this instruction manual before installation and use.
2.Do not touch the power plug with wet hand.
3.Switch off the main power when the unit is not in use. (Disconnect the power plug from the power outlet when you do not intend to use the unit for a prolonged period of time.) Before moving the unit, remove the disc from the unit/ tray first.
4.Do not open the cover or touch any of the components exposed out of the unit, only for qualified technicians.
5.Do not expose the unit under direct sunlight or nearby objects that radiate heat to avoid damage the unit.
6.Do not place the unit in moist and humid conditions, which effect the pick-up of the unit lead to out of work.
7.Place the unit on a flat surface and ventilative condition. Please ensure the ventilation holes are not covered to avoid overheating and malfunction.
8.Clear the panel and case with soft dry cloth only, do not apply any kind of thinner, alcohol or sprays.                                                                                                     9. The apparatus shall not be exposed to dripping or splashing and that no objects filled with liquids, such as vases, shall be placed on the apparatus

10. unplug the apparatus when not using for a long time oriented to disconnect from the Mains power

11. No naked flame sources, such as lighted candles, should not be placed on the apparatus.

12.Make sure there is free circulation of air (do not place the device on shelves, on a thick carpet, on a bed, or anywhere that obstructs the ventilation slots, and leave at least 10cm clearance on all sides.

13.The apparatus shall not be exposed to dripping or splashing.

14.The mains plug is used as the disconnect device, the disconnect device shall remain readily operable.

15.The battery (battery of batteries of battery pack) shall not be exposed to excessive heat such as sunshine, fire or the like.

16. The mains plug used as the disconnect device,disconnect device shall remain readily operable.

If the electric cable is damaged, it must be replaced by the manufacturer, or by the qualification persons, in order to avoid a danger.

The product's function maybe reset when it is subjected to electrostatic discharge

disturbance. Just follow the instruction described in this manual to set the function again

for normal operation.

During electrostatic discharge and the strong electromagnetic field, the product may malfunction, then press “POWER”button and the product will return to normal performance.

Macrovision:

This product incorporates copyright protection technology that is protected by method claims of certain U.S patents and other intellectual property rights owned by Macrovision Corporation, and is intended for home and other limited viewing uses only unless otherwise authorized by Macrovision Corporation. Reverse engineering or disassembly is prohibited.

Manufactured under license from Dolby Laboratories.

"Dolby" and the double-D symbol are trademarks of Dolby Laboratories.

Mains plug is used as the disconnect device. It shall remain readily operable, and should

not be obstructed during intended use.

Please do not use USB extension cords longer than 25cm.

1. PRECAUTIONS

Before using this unit, be sure to read the applicable items of the operating instructions and these safety suggestions carefully.

Afterwards keep them handy for future reference.

Take special care to follow the warnings indicated on the unit itself as well as in the operating instructions.

## Safety

Power Source-The unit should be connected to power supply only of the type described in the operating instructions or as marked on the unit.

No naked flame sources, such as lighted candles, should not be placed on the apparatus.

Make sure there is free circulation of air (do not place the device on shelves, on a thick carpet, on a bed, or anywhere that obstructs the ventilationslots, and leave at least 10cm clearance on all sides)

The apparatus shall not be exposed to dripping or splashing.

Environment

- Water and moisture do not use this unit near water.

For example, near a bathtub, wash bowl, swimming pool, or the like, Damp basements should also be avoided.

- Condensation-moisture may form on the lens in the following conditions.

Immediately after a heater has been turned on.

In a steamy or very humid room.

When the unit is suddenly moved from a cold environment to a warm one, if moisture forms inside this unit, it may not operate properly, to correct this problem wait about one hour for the moisture to evaporate.

- Heat never use or place this unit near a heat source.

Do not leave it in an automobile exposed to direct sun light for a long time with the doors and windows closed as this may deform the cabinet.

### 2. FRONT PANEL AND REAR PANEL ILLUSTRATION

| 1. Open/Close 2. Play/Pause 3. STOP 4. POWER | 5. Disc Tray 6. Sensor 7. Tray Display Screen 8. USB |
| --- | --- |

NOTE:The USB port is used as a direct connective port. Please don't use any extended connective Cords.

Rear panel

| 1. Downmix L 2. Component Video Output 3. Composite Video Output 4. 5.1 Channel Audio Output | 5. Downmix R 6. HDMI 7. Coaxial Output 8. S-video Output |
| --- | --- |

3. REMOTE CONTROL

1. STANDBY

2. USB

3. AUDIO

4. SUB/MP3

5. CLEAR

6. SETUP

7. UP button

8. LEFT button

9. DOWN button

10.REV

11.FWD

12.STEP

13.STOP

14.VOLUME +/-

15.L/R

16.N/P

17.SLOW

18.VIDEO OUT

19.HD/RES

20.OPEN/CLOSE

21.SEARCH

22.Numeric Button

23.MENU

24.TITLE

25.DISPLAY

26.RIGHT button

27.ENTER button

28.NEXT

29.PREV

30.PLAY/PAUSE

31.A-B

32.REPEAT

33.MUTE

34.ANGLE

35.PBC

36.RETURN

37.ZOOM

38.PRGORAM

Using the Remote Control

Unless stated other wise, the remote control can operate all the features of the DVD Player, Always point the remote control directly at the remote sensor on the front of the DVD Player, not the TV.

Battery Installation

Remove the battery compartment lid on the rear of the remote control by sliding the lid down, then off.

Place two AAA batteries inside the battery compartment with their + and ends aligned as indicated.

Do not mix old and new batteries or different types of batteries

Replace the battery compartment lid. Slide the lid until you hear it click into place.

##### Important:

When replacing batteries, please take care to waste them in accordance with current regulation. For this matter please put them in the right assigned place, in order to ensure they are surely recycled while respecting environment. Do not burn them or burry them.

4.CONNECTIONS

Before connecting, make sure to turn off power of all apparatus.

After finishing connection , turn on the power again.

AV cable(accessory) should be connected by color.

Yellow plug ------------video terminal (yellow)

White plug-------------left audio terminal (white)

Red plug---------------right audio terminal (red)

Must connect properly, especially the plug should be inserted to the bottom.

Notice: this picture is only for your reference, all the functions should be based on the actual machine.

##### DVD+TV

Notes :

There are the following ways of connecting the player AV output to the TV set:

- S-video cable and left/right audio cable (set the video type to component)

- Composite video cable and left/right audio cable

- Component video cable and left/right audio cable (set the video type to component, if you connecting a progressive scan TV set, please set the video type to P-Scan)

- HDMI cable(press V-MODE button to switch the video mode to HD and set the video type to HD)

DVD+ AV Amplifier:

Notes :

There are the following ways of connecting the player with amplifier:

- left/right audio output

- coaxial audio output

Warning:

Do not connect any amplifier with coaxial connector that does not contain Dolby digital AC 3 or DTS decoder. Any attempt to do so and play of DVD disc will result in damaging your speakers and causing possible hearing injury. Other than DVD disc, VCD and CD can be operated normally without conforming to the above restriction.

# Setting of TV system

If you find the image like non-fully zoomed picture, brim picture covered or black & white picture etc, please select PAL or NTSC by systems setting or press N/P button to switch the TV system on the remote control. This DVD disc is capable of supporting different scales picture, as a result, non-full zoomed picture could appear or picture be partially covered on user’s TV set.

5. FUNCTION OPERATION

5.1 Function Buttons On the Front Panel

These button’s function is the same as the corresponding buttons on the remote control. For the detail information, please see the following texts.

5.2 Function Buttons On the Remote Control

Menu Play (compatible with DVD discs)

Press Menu Button to display the DVD Root Menu.

When menu display, you can press Arrow buttons to highlight the menu item you want to choose.

Press Enter, you can begin to play with the highlighted item.

Pbc  Play (compatible with SVCD, VCD2.0 discs)

Press Menu Button to display the SVCD or VCD 2.0 menu.

When menu display, you can press Arrow buttons to highlight the menu item you want to choose.

Press Enter, you can begin to play with the highlighted item.

Title Play (compatible with DVD discs only)

During DVD playback, press Title button to display the DVD title content.

Sequence Play (compatible with DVD, SVCD, VCD, WMA, MP3/JPEG discs)

When a SVCD or VCD disc is loaded and MENU is on, once MENU button pressed, MENU switches off. Then the player can play in sequence playback mode; when MENU button pressed twice, MENU switches on.

Except for SVCD or VCD discs, in the stop state, when press Playpause button, the player then enters sequence playback mode. This feature is effective to all the compatible discs.

Program Play (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

The Program Playback option allows you to enter the order in which you want chapters or tracks to play, maximum to 16.

Program Menu:

Press PROGRAM button to enter program playing state.

Using ARROW key to highlight the item.

Using numeric key to enter chapter/track.

Using Play/Pause to start play.

Notes:To clear the programmed order, use Arrow buttons to highlight the “CLEAR” item, then press Enter.

To modify the programmed order, use Arrow buttons to highlight the modifying item and enter the new chapter / track you want to program directly.

Selecting Tracks (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

Press Numeric buttons to select desired title/ tracks.

Notes: To select tracks from 1 to 9, press corresponding Numeric buttons (1-9, 10).

When selecting track of 10 or above, press 10+ button first.

Skipping Tracks (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

To skip forward to the beginning of the next title / chapter / track on a disc, press Next

To skip backward to the beginning of the preceding title / chapter / track on a disc, press Prev.

Multispeed Fast Motion Playback  (compatible with DVD, SVCD, VCD, CD, WMA, MP3)

Press the F.F button to fast forward play.

Fast Forward Playback has four speeds: × 2, × 4, × 8, × 20.

Press the F.R button to fast reverse play. Fast Reverse Playback has four speeds: × 2, × 4, × 8, × 20.

Note :Press Playpause button to resume normal playback.

Multispeed Slow Motion Playback  (compatible with DVD, SVCD, VCD)

Press Slow button to enter Slow Motion Playback mode.

Slow Motion Playback has six speeds: 1/2, 1/3, 1/4, 1/5, 1/6, 1/7.

Press Playpause button to resume normal playback.

Freeze Frame and Frame Advance  (compatible with DVD, SVCD, VCD, CD,HD-CD, WMA, MP3/JPEG discs)

During CD, HD-CD, WMA or MP3 / JPEG playback, press Playpause once to freeze playback; Press the Playpause button again or Playpause button to resume normal playback.

During DVD, SVCD or VCD playback, press Playpause once to freeze playback; Press of the Playpause button again to advance the picture one frame at a time; Press Playpause to resume normal playback.

Using the Repeat Feature (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

While a disc is playing, press Repeat button to switch repeat mode.

DVD playback:  Off, Chapter Repeat, and Title Repeat.

VCD playback:  Off, chapter Repeat, and Disc Repeat.

Using the Display Feature  (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

Press Display to show playing information.

Changing the Audio Language  (compatible with DVD, SVCD discs)

Press Language to change the audio language.

Changing the Camera Angle (compatible with DVD discs)

Press Angle to switch camera angle.

Changing the TV System (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

Press N/P button to change TV system: NTSC, PAL, and AUTO.

Zoom In the Video Image (compatible with DVD, SVCD, VCD, JPEG discs)

The Zoom Function allows you to enlarge or zoom out the video image .zoom factor: Zoom 2, Zoom 3, Zoom 4, Zoom 1/2, Zoom 1/3, Zoom 1/4, Zoom Off.

Use the Arrow buttons to pan across the screen when the zoom factor is 1, 2 or 3.

Repeat a Specific Segment on the Disc that You Define (compatible with DVD, SVCD, VCD, CD, WMA, MP3 discs)

During the normal or fast motion playback, press the A-B button once, and “REPEAT A-” will appear on the screen. This indicates the start of the loop to be repeated.

During the normal of fast motion playback, press A-B again, and “REPEAT A-B” will appear on the screen. It indicates the end of the loop.

Volume control (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

You can press the Volume button on the remote controller or the TV set or the amplifier to control the sound level.

Note: During DVD PCM output playback, this feature does not work.

HD/Res.(HD-Mode)

Press the button to switch HD video output resolution

Note:

Under HD mode, this button is used to select the output resolution.

Your DVD player can generate some resolutions that may not be supported by your TV set. If this happen ,your TV screen will probably turn black. To solve the problem, please repeatedly press “HD/Res.”  button until you select a resolution that can be well supported by your TV set.

Mute the Audio Output (compatible with DVD, SVCD, VCD, CD, WMA, MP3/JPEG discs)

Pressing the Mute button allows you to reduce sound to an inaudible level. Press the button again to normal condition.

Subtitle  (compatible with DVD)

Press the Subtitle button to switch subtitle language or subtitle off .

6.  Playing WMA, MP3 and JPEG discs

Place a MP3 / JPEG disc into the tray, and the player will auto search the directories saved MP3 / JPEG files. After searching, it will auto go to the first directory saved MP3 files (to the disc containing both mp3 and jpeg files; To the disc containing single type of files, it will auto go to the first directory) and play the first song in the directory (“01:” is highlighted).The screen displays as the picture below:

Use the Arrow buttons to enter the Directory Region and highlight the desired directory, then the first track under it starts playback; Also you can use the Arrow buttons to enter the File Region and select the file you want to play.

- Press the Left and Right arrow buttons, you can activate Directory Region, File Region and File Switch Region separately.

- The highlight in Directory Region and File Region is shown as the picture above; While it appears a transparent rectangle in File Switch Region.

- In File Switch Region, you can press the Left and Right arrow buttons to switch the highlight, then press PLAY; the “M”, “P”, and “V” button is illustrated in the above picture. For example, if a disc with MP3 and JPEG files but without any NES files is loaded, the color of the letter “M” and “P” will appear to white and “V” black. If a MP3 file playing, “M” turns darker.

- To the disc with WMA, MP3 and JPEG files, when playing a JPEG picture, press the Return button to return to JPEG list.

- When a picture playing or pausing, press the Arrow buttons to change the display angle:

Up: flip vertical

Down: flip horizontal

Left: rotate 90O counter-clockwise

Right: rotate 90O clockwise

- When a picture stop, press the Menu button, the screen displays small pictures. Up to 6 small pictures are displayed per screen. To view the next  6 pictures, press the Next button or the RIHGT button to go forward and the Prev button or the LEFT button to go to the previous page.

- You can press the Numeric buttons to select the file you want to play. For example, if you desire the 15th track in current list, press the “10+” button once, then press “5”, the 15th starts playback.

Notes: When playing a disc with MP3 files, video cannot be output while only digital audio can.

- When playing JPEG pictures, press the Program button to choose the pictures switched mode. There are 17 switch modes; Press Program repeatedly to activate the function and choose the required switch mode: Spread from Up to Down, Spread from Down to Left, Spread from Left to Right, Spread from Right to Left, Spread from Left Up to Right Down, Spread from Right Up to Left Down, Spread Left Down to Right Up, Spread from Right Down to Left Up, Spread from Center to Up / Down, Fold from Up / Down to Center, Fold from Left / Right to Center, Horizontal Shutter, Vertical Shutter, Fold from the Corners to the Center, Random Play, Off.

7. CD-RIPPING

Press the SUB/MP3 button on the remote control, the following interface will be display.
Use the direction button(UP,DOWN,LEFT,RIGHT), you can select your desired options displayed in the interface

Press the ENTER button repeatedly, you can select different speed, different bitrate, create or cancel palylist.

Use the DOWN direction button ,you can select TRACKS option, the following interface will be displayed.

On the top of the picture, press the ENTER button repeatedly, you can select all the tracks, and press again, you can unselect all the tracks.

Use the DOWN and UP direction button, and press ENTER button, you can select or unselect your desired tracks.

Use the DOWN direction button ,you can select START option, the following interface will be displayed.

Press the ENTER button, the CD-RIPPING will begin. In the CD-RIPPING progress, you can change the speed by

pressing ENTER button and also you can cancel the current operation.

Use the DOWN direction button ,you can select QUIT option, the following interface will be displayed.

Press ENTER button, you can exit the CD-RIPPING interface.

8. Setup Menu System

### General Steps of Setup Menu

Press the Setup button, the main setup menu appears.

When the main menu displayed, the DVD system will auto pause.

Press the Left and Right arrow buttons to highlight the icon of the desired page, then press the Down or Right arrow button to activate that page;

Press the Arrow buttons to highlight the Exit Setup option, then press Enter or Setup again to exit Setup Menu.

Notes:

When setup, the pressing of the following buttons will turn off the setup menu, while the choices that have been changed will be saved: N / P, Subtitle, F.F, F.R, Vol +/-, , Title, , Play/Pause, Stop, , Step, Next, Prev, Zoom, , Program, Repeat, A-B, Angle, Language, Display, Mute, Setup.

Setup Items Introduction

### 8.1  SYSTEM SETUP

The system setup options are illustrated below:

8.1.1  TV SYSTEM

According to the Color System of the TV, you can choose the TV System.

AUTO: Choose this settings if your DVD player is connected to a multi-system TV.

NTSC: Choose this settings if your DVD player is connected to a NTSC TV.

PAL: Choose this settings if your DVD player is connected to a PAL TV.

8.1.2  SCREEN SAVER

If the screen saver is on, when your DVD player is in Stop, Open, or No Disc state, or there is a frame frozen for more than 3 minutes, the screen saver will be displayed. This feature can protect the screen from damage.

ON: Choose this setting to activate the screen saver.

OFF: Choose this setting to cancel the screen saver

### 8.1.3 HD OUTPUT

In this option, you can select different resolution. Under this condition, the VIDEO and TV TYPE are both invalid. The default set is HD OFF.

When you change to the HDMI output with different resolution from the normal video output, an interface will be displayed. Select “yes” to confirm change, select “no” to exit.

8.1.4  VIDEO

Interlace-YUV: Choose this setting if your DVD player is connected to a TV with Y, Cb, Cr video cables.

P-SCAN YPBPR: Choose this setting if your DVD player is connected to a progressive scan TV with Y, Cb, Cr video cables.

S-VIDEO: Choose this setting if your DVD player is connected to TV with S-Video cable.

Notes:

When your DVD player output composite video, you can choose the setting randomly.

Choose P-SCAN if your DVD player is connected to a progressive scan TV.

8.1.5  TV TYPE

16 : 9 (Wide Screen): Choose this setting if your DVD player is connected to a wide screen TV. If you choose this setting and you don’t have a wide screen TV, the images on the screen may appear distorted due to vertical compression.

4 : 3 LB (Letter Box): Choosing this setting if your DVD player is connected to a normal ratio TV. connector You’ll see the movies in their original aspect ratio (height-to-width ratio). You’ll see the entire frame of the movie, but it will take up a smaller portion of the screen vertically. The movie might appear with black bars at the top and bottom of the screen.

4 : 3 PS (Pan & Scan): Choosing this setting if your DVD player is connected to a normal ratio TV. You can fill the movie to the entire screen of your TV. This might mean that parts of the picture (the left and right edges) won’t be seen.

Notes:

The image display ratio is related to the recorded disc format. Some disc can not display images in the aspect ratio you choose.

If you play a disc recorded in 4:3 format on a wide screen TV, black bars will appear on the left and right side of the screen.

You must adjust the screen setting depending on the type of the television you have.

8.1.6  PASSWORD

The password option is initialized locked, and you cannot set the ratings limit or change the password. In order for the Ratings feature work, the password mode must be turned on. If you want to set the ratings limit, you will need to enter the default password, which is 0000, then press Enter to confirm. To change the password, you will be prompted for the old password, then be prompted for a new. Enter a 4-digit number (this is your password).

8.1.7  RATING

The Rating feature is a rating limit system, like movie ratings. It works with DVD discs that have been assigned a rating. This helps you control the types of DVDs that your family watches.

There are these rating options: 1 KID SAFE,2 G,3 PG,4 PG13,5 PG-R,6 R,7 NC-17,8 ADULT.

Notes:

If the password option is locked, you cannot set the ratings limit; the rating limit doe not work when the password option unlocked.

8.1.8  DEFAULT

Choose this option to resume all the setup options to default settings.

### 8.2  LANGUAGE SETUP

The language setup options are illustrated below:

8.2.1 Highlight the OSD LANGUAGE option, and

press the Arrow buttons to choose the OSD language you prefer. Press Enter to confirm, and it

will display OSD in that language.

8.2.2 Highlight the AUDIO LANG option, and press the Arrow buttons to choose the audio language you prefer. Press Enter to confirm. If the disc you are playing has that language available, it will output audio in that language.

8.2.3 Highlight the SUBTITLE LANG option, and press the Arrow buttons to choose the subtitle language you prefer. Press Enter to confirm. If the disc you are playing has that language available, it will displays subtitles in that language.

8.2.4 Highlight the MENU LANG option, and press the Arrow buttons to choose the menu language you prefer. Press Enter to confirm. If the disc you are playing has that language available, it will displays menu in that language

Notes:

If your preferred language isn’t available, the disc’s default language will be played.

8.2.5 Highlight the MPEG4 SUBTITLE option, and press the Arrow buttons to choose the Subtitle you prefer. Press ENTER button to confirm.

Notes:If your preferred language isn’t available, the disc’s default language will be played.

### 8.3  AUDIO SETUP

The audio setup options are illustrated below:

8.3.1 Highlight the AUDIO OUT option, and press the Arrow buttons to choose the audio output mode you prefer. Press Enter to confirm; There are 3 optional audio output modes: SPDIF/OFF, SPDIF / RAW, and SPDIF/ PCM.

SPDIF/OFF : From the Coaxial Output, no signal is output.

SPDIF/ RAW: Choose this setting if your DVD player is connected to a power amplifier with a coaxial cable When playing a disc recorded with Dolby Digital, DTS and MPEG audio formats, the corresponding digital signals will be output from the Coaxial Output jack Output jack. This player’s digital output jacks are designed for a connection to a Dolby Digital, DTS , or MPEG receiver or decoder.

SPDIF / PCM: Choose this setting if your DVD player is connected to a 2-channel digital stereo amplifier; When playing a disc recorded with Dolby Digital and MPEG audio formats, audio signals will be modulated into a 2-channel PCM signal, and it will be output from the Coaxial Output jack Output jack.

8.3.2  GM5

  - GM5 audio effect function could process 2CH audio source into 5CH sound. The different modes are achieved by different DSP operating method, and the effects will be different, too.

  - GM5 can support CDDA/MP3/VCD/AC3 2CH audio source.

    - There are extended items of GM5 function: SONG/CINEMA/OFF

    - GM5 will be unable when it is set to OFF.

    - When it is set to SONG or CINEMA, the corresponding audio effect will be achieved by GM5 function.

8.3.3  KEY

Use the Up and Down arrow button to move the scroll and adjust the key.

8.4  VIDEO SETUP

The video setup options are illustrated below:

8.4.1 DEEPCOLOR：OFF/30 BIT/36 BIT

8.4.2  BRIGHTNESS

Use the Up and Down arrow button to move

the scroll and adjust the brightness.

8.4.3  CONTRAST

Use the Up and Down arrow button to move the scroll and adjust the contrast.

8.4.4  HUE

Use the Up and Down arrow button to move the scroll and adjust the hue.

8.4.5  SATURATION

Use the Up and Down arrow button to move the scroll and adjust the saturation.

8.4.6  SHARPNESS

Use the Up and Down arrow button to move the scroll and adjust the sharpness.

8.5  SPEAKER SETUP

The speaker setup options are illustrated below:

8.5.1  DOWNMIX

Use the Up and Down arrow button to move the scroll and adjust the saturation.

LT / RT: Choose this setting, it will be sound like

in a movie theatre if the original movie the disc was recorded from was encoded in the Pro Logic Dolby Digital format.

STEREO: when 2-channel sound playing, stereo signal will be output.

VSS: Choose this setting, when a disc recorded with 5.1-channel sound format playing, the

fictitious surround soundtracks effect will be heard from the main soundtrack.

5.1CH:Choose this setting to output the 5.1-channel sound.

### 8.6  DIGITAL SETUP

The video setup options are illustrated below:

8.6.1 OP MODE

LINE OUT: Choose this setting to line out the input signal. It will be more suitable for playing in the night time, for the output volume is low.

RF REMOD: It will be more suitable to choose this setting for playing in the day time, for the output volume is high.

8.6.2 DYNAMIC RANGE: When “LINE OUT” is selected, choose this setting to adjust the line out ratio and get different effect. If adjusting to FULL, the audio signal peak value will be the minimum; while adjusting to OFF, maximum.

NOTE:

This option will be validated when the DOWNMIX OFF.

8.6.3 DUAL MONO (LEFT + RIGHT)

Choose this option to setup the L and R mono output mode; There are four modes in this option: STEREO, MONO L, MONO R, and MIX MONO.

9. Specification

| Signal system | PAL/AUTO/NTSC |
| --- | --- |
| Frequency Response | 20HZ-20KHZ( at 1KHz 0 dB) |
| Signal-to-Noisy Ratio | ≥90 dB( at 1KHz 0 dB) |
| Audio Distortion +Noise | ≤-60dB ( at 1KHz0dB) |
| Audio Output(Analog audio) | 2.0+0-1..0Vrms |
| Video Output | Composite:1.0±0.2Vp-p Load impedance: 75Ω, imbalance, negative polarity |
| MAX Resolution | >500 lines |
| Power Supply | ~110V-240V 50 Hz /60Hz |
| Dimensions | 255(mm)X220(mm)X38(mm) |
| Supported Disc | MPEG-4/Xvid/DVD/SVCD/VCD/CD/MP3/Kodak Picture/Photo CD |

REFERENCE:

To protect your DVD player, the interval between two consecutive Power button operations must be above 30 seconds.

No using for a period of time, please pull out the plug from the electronic outlet.

The player should be operated only on condition that the type of power supply corresponds to the marking label. Otherwise, it doesn’t work or even be damaged.

The player may not be compatible with some encrypted discs.

If the player stops playback by accident, please power it on again.

10.Troubleshooting

No picture:

Ensure that the equipment is connected properly.

Connect the video cable securely into the appropriate jacks.

Select the appropriate video input mode on the TV

### Picture Distorted

The disc is dirty and needs to be cleaned.

It is normal for some distortion to appear while Fast Motion Playback.

### No Sound

Check the connections

Check the volume on the equipment you have connected.

Connect the audio cable securely into the appropriate jacks.

Ensure that the input settings for the television and stereo system are correct.

If the player is in mute mode, please press Mute button again to turn on sound.

### No Stereo Sound Effect when playing video CD or CD

Check the connections

### The Remote Control does not work properly

Remove the obstacle existing between the DVD player and the remote control.

The remote control is too far from the DVD video player. Operate the remote control within operating distance.

Point the remote control at the remote sensor of the DVD video player.

If the batteries in the remote control are exhausted, please replace batteries with new ones.

### The DVD player does not start playback

No disc is inserted.(no disc symbol appears on the screen). Please insert disc.

The disc is place upside down. Place the disc with the playback side down.

The disc is dirty. Please clean the disc.